Question 1: Definition of Discrete Time Signal

Which of the following best defines a discrete-time signal?

A) A signal defined at continuous intervals of time.

B) A signal defined only at discrete instants of time.

C) A signal having infinite amplitude at all points.

D) A signal with zero frequency spectrum.

Correct Answer: B

### Question 1: Linearity and Circular Time Shifting

Consider a discrete-time sequence xnof length Nwith a DFT Xk.

Statements:

- The DFT is a linear transform, meaning the DFT of a⋅x1n+b⋅x2nis equal to a⋅X1k+b⋅X2k, provided both sequences are of the same length N.

- Shifting a sequence circularly in the time domain by msamples, xn-mN, results in multiplying its DFT by e-j2πNkm.

- Circular time-shifting alters both the magnitude spectrum and the phase spectrum of the sequence.

Which of the statements are correct?

- A) 1 and 2 only

- B) 2 and 3 only

- C) 1 and 3 only

- D) 1, 2, and 3

Question 3: Linearity of Systems

A system is defined to be linear if it satisfies which of the following principles?

A) Superposition and Homogeneity

B) Time Invariance and Causality

C) Stability and Memorylessness

D) Periodicity and Symmetry

Correct Answer: A

Answer: A

- Explanation: Statements 1 and 2 are fundamentally true properties of the DFT. Statement 3 is incorrect because multiplying Xkby a complex exponential e-j2πNkmonly introduces a linear phase shift; it leaves the magnitude spectrum Xkcompletely unchanged (e-jθ=1).

### Question 2: Conjugate Symmetry and Real-Valued Signals

Let xnbe an N-point real-valued sequence, and Xkbe its N-point DFT.

Statements:

- The DFT Xkexhibits conjugate symmetry, meaning Xk=X*N-kNfor 0≤k≤N-1.

- The real part of Xkis an even function of k, and the imaginary part is an odd function of k(modulo N).

- The value of the DFT at k=0, X0, must always be a purely real number.

Which of the statements are correct?

- A) 1 and 2 only

- B) 1 and 3 only

- C) 2 and 3 only

- D) 1, 2, and 3

Answer: D

Question 5: Sampling Theorem

According to Nyquist-Shannon sampling theorem, what minimum sampling frequency fs is required to avoid aliasing for a signal with maximum frequency fm?

A) fs >= fm

B) fs >= 2 * fm

C) fs = fm / 2

D) fs = 0.5 * fm

Correct Answer: B

### Question 3: Circular Convolution vs. Linear Convolution

Let X1kand X2kbe the N-point DFTs of two N-point sequences x1nand x2n, respectively.

Statements:

- Multiplying the DFTs, Yk=X1k⋅X2k, corresponds to the linear convolution of x1nand x2nin the time domain.

- To compute an L-point linear convolution using DFTs where x1nis of length N1and x2nis of length N2, the DFT size must satisfy N≥N1+N2-1.

- Circular convolution is commutative, meaning x1n⊛x2n=x2n⊛x1n.

Which of the statements are correct?

- A) 1 and 2 only

- B) 2 and 3 only

- C) 1 and 3 only

- D) 1, 2, and 3

Answer: B

- Explanation: Statement 1 is false because multiplication in the frequency domain corresponds to circular convolution (x1n⊛x2n), not linear convolution. Statement 2 is true (zero-padding to N1+N2-1prevents time-domain aliasing). Statement 3 is true as circular convolution obeys commutative laws.

Question 7: BIBO Stability

A discrete-time LTI system is Bounded-Input Bounded-Output (BIBO) stable if and only if its impulse response h(n) satisfies which condition?

A) It is absolutely summable.

B) It is strictly zero for all negative time.

C) It has infinite energy.

D) It is an odd symmetric function.

Correct Answer: A

### Question 4: Parseval's Relation and Energy

Let Xkbe the N-point DFT of a complex sequence xn.

Statements:

- Parseval’s theorem states that the total energy calculated in the time domain equals the total energy calculated in the frequency domain, without any scaling factors.

- Mathematically, the relation is represented as n=0N-1xn2=1Nk=0N-1Xk2.

- If a sequence is scaled such that yn=2xn, the total energy in the DFT domain increases by a factor of 4.

Which of the statements are correct?

- A) 1 and 2 only

- B) 2 and 3 only

- C) 1 and 3 only

- D) 1, 2, and 3

Answer: B

Question 9: Energy vs Power Signal

What is the total average power of a finite-energy signal?

A) Infinity

B) Zero

C) One

D) Negative infinity

Correct Answer: B

- Explanation: Statement 1 is false because there is a scaling factor of 1/Nrequired due to the definition of the forward/inverse DFT. Statement 2 correctly shows this 1/Nfactor. Statement 3 is true because energy depends on the square of the magnitude (22=4).

### Question 5: Duality Property

The duality property reflects the symmetrical relationship between the forward DFT and the Inverse DFT (IDFT).

Statements:

- If the DFT of xnis Xk, then the DFT of the sequence Xnis N⋅x-kN.

- The duality property allows us to compute an IDFT using a forward DFT algorithm by reversing the output sequence index (modulo N) and scaling.

- If a signal is purely imaginary in the time domain, its DFT will always be purely imaginary in the frequency domain.

Which of the statements are correct?

- A) 1 and 2 only

- B) 2 and 3 only

- C) 1 and 3 only

- D) 1, 2, and 3

Answer: A

- Explanation: Statements 1 and 2 accurately define and apply the duality property. Statement 3 is false; if xnis purely imaginary, its DFT Xkexhibits conjugate anti-symmetry (Xk=-X*N-k), which means its real part is odd and its imaginary part is even, but it is not purely imaginary unless the signal also possesses specific time-domain symmetry (like being anti-symmetric).